## Project Title: IoT-Based Health Assist Robot

## To design and build a hospital-assistive robot that can deliver medicines, food, and water to patients — especially useful for cases where direct human contact should be minimized (such as infectious diseases or isolation wards).This robot is autonomous and IoT-enabled, capable of dispensing medicine at specific times and transporting food or water on command.

## Main Functions:

## Medicine Dispenser:

- Automated system with two servo motors.

- Each servo corresponds to a set of compartments (like trays for different medicines).

- Example:

- Servo 1 controls compartments 1–7.

- Servo 2 controls compartments 8–14.

- The servos rotate to the right angle (0°, 30°, 60°, etc.) to dispense a specific medicine.

- User inputs (through serial monitor or switch) which compartment to open (1–14).

- The corresponding servo(s) rotate to align and dispense the medicine.

## Food & Water Compartments:

- The robot has two separate compartments:

- Food compartment with a heating pad (to keep food warm).

- Water compartment with insulation (to maintain room temperature).

- Each compartment can open using a servo-based door mechanism (simulated by rotating from 0° to 180° in Tinkercad) Servo-controlled doors open when triggered (manually or via program).

## Movement System:

## The robot moves on 4 geared DC motors (like an RC car).

## Controlled via an L298N motor driver connected to an ESP32 or Arduino.

- Can be made to move forward, backward, left, or right, either autonomously (later) or manually via buttons/commands.

- Safety & Support:

- IR or Ultrasonic sensors can be used to detect obstacles and stop movement automatically.

- Optional: Camera (ESP32-CAM) for remote navigation or basic vision tracking.

Components

- Acrylic sheets [6] (3*5mm, 12*12 inch)

- Dimensions: width=20cm

  - Length=30cm

- 10cm-----medicine dispenser

- 12cm-----water & food

- 6cm------first aid

- 4cm------circuit

- 5cm-------wheels

- 9v hub motors = 4

- 4 wheels

- 2 breadboards

- 12v li ion battery with charger

- jumper wires

- screws clamps hinges

- ft232rl

- esp 32

- esp 32 cam module

- mg996r 180 degree = 2

- mg90 s = 2

- l298n

- dht11

- ds18b20

- ESP32 / Arduino → Controls all logic

- L298N Motor Driver → Drives 4 DC motors

- Servo Motors → For dispenser + door mechanisms

- Relay Module → Controls heating pad

- Voltage Regulator (LM2596) → Steps down 12V → 5V

- Common GND → Connect all grounds together for stable reference

### 1. Base & Wheels

- Start with the chassis/base plate (your acrylic sheet, 20×30 cm).

- Attach the 4 hub motors to the base corners.

  - Keep the wheels aligned straight.

  - Make sure the wheel overhang doesn’t interfere with compartments.

- This gives you a stable mobile platform before adding anything else.

### 2. Motor Driver & Power Circuit Placement

- Decide where your 4 cm “circuit” compartment will go (you mentioned 4 cm).

  - Place it near the center of mass so the robot doesn’t tip.

  - This compartment will house:

    - Motor drivers (H-Bridge or ESCs)

    - Microcontroller

    - Power supply

- Keep wires from motors to the driver as short as possible to reduce voltage drop

### 3. Compartment Placement

- Stack compartments from front to back according to your sizes:

  - Medicine dispenser (10 cm) – front or center.

  - Water & Food (12 cm) – next, slightly larger, could be center or back.

  - First Aid (6 cm) – either front/top depending on access.

- Make sure access doors or dispensers can open without hitting wheels or other compartments.

- Use screws or glue to secure compartments.

### 4. Microcontroller & Sensors

- Microcontroller goes in the circuit compartment.

- Any distance/IR sensors can be mounted at the front and corners for navigation.

- Servo motors for dispensing can be mounted inside the compartments.

### 5. Battery Placement

- Place the battery low and central to keep center of gravity stable.

- Secure it with a strap or tape inside the circuit compartment or below the base plate.

### 6. Wiring Planning

- Route motor wires first, then power wires, then control wires.

- Keep high-current motor wires separate from sensor/wire signals to avoid noise.

- Label wires if possible to make troubleshooting easier.

Rule of Thumb: Always assemble heaviest components first (motors, battery) to prevent tipping, then stack the lighter ones (dispensers, circuit boards).

Bottom layer:

- Wheels + motor mounts (9V hub motors × 4)

- Attach motors using clamps & screws to the acrylic base

- Ensure wheels fit under 20×30 cm base and rotate freely

Second layer:

- Circuit board area (ESP32, L298N, breadboard, battery)

- Size: 4 cm

- Keep this in front or center for easy maintenance

- Mount battery with clamp

- Keep common GND connection nearby

Third layer: Medicine dispenser (10 cm)

- Place MG996R × 2 to control 14 compartments

- Mount servos with screws & brackets

- Hinged tray doors or sliding compartments

Fourth layer:

- Food & Water compartments (12 cm)

- Place MG90S × 2 to control doors

- Food compartment: add heating pad (relay-controlled)

- Water compartment: can stay insulated

Top layer:

- Optional first-aid compartment (6 cm)

- Small storage area for emergency supplies

- Tip: Keep enough vertical clearance so servos can rotate freely and Peltier or heating elements don’t touch acrylic.

Optional sensors for safety:

- IR or Ultrasonic at front & sides → Stop motors on obstacle detection

- ESP32 reads sensor → stops L298N motor driver outputs

Actuators:

- Heating pad → Relay → ESP32

- Peltier → Relay or MOSFET → ESP32 (optional for cooling food/water)

Logic:

- Food compartment: maintain 37–45°C → relay switches heating pad ON/OFF

- Water compartment: maintain 25–30°C → Peltier if needed

## Assembly Tips

## Fix acrylic sheets in order using screws & clamps

- Use hinges for medicine tray doors and food/water lids

- Arrange electronics in breadboard / mounts before permanent soldering

- Test servo & motor operation individually before final assembly

- Keep wiring neat → zip ties & cable clamps

## L298N motor driver

- The L298N is a dual H-bridge motor driver IC.

- For driving DC motors or small gear motors

- It can control the direction and speed of two DC motors independently.

- You can control speed using PWM (Pulse Width Modulation) signals from your microcontroller (like Arduino, NodeMCU, Raspberry Pi, etc.).

- Each motor is controlled by an H-bridge (because the circuit looks like the letter H).

+V

|

S1      S2

|------|

|      |

Motor

|      |

S3      S4

|

GND

- When S1 and S4 are ON → current flows one way → motor spins forward.

- When S2 and S3 are ON → current flows opposite way → motor spins backward.

- When all are OFF → motor stops.

- When both S1/S2 or S3/S4 ON → motor brakes (both ends tied to same potential).

In your Health Assist Robot, the L298N will be part of the motion/navigation subsystem.

- Input: Control commands from your main microcontroller (manual joystick or autonomous navigation algorithm).

- Output: Drives the left and right wheel motors.

- Integration: Works with sensors (ultrasonic, IR, encoder) for obstacle detection & speed feedback.

- You can use:

- PWM to vary speed (smooth movement in corridors)

- Direction control for turning, reversing, etc.

- Combine it with encoder feedback for accurate distance/speed control.

## 9V Hub Motor

Hub motors are compact, give direct drive to the wheel (no gearbox), and often have high torque at low speeds—useful for a small robot that must push a tray.

ESP32 is a powerful microcontroller

Built-in Wi-Fi and Bluetooth

## Using 12 V Li-ion with Your Health Assist Robot.

DHT11 is a temperature & humidity sensor.

- It gives a digital signal directly to your microcontroller (ESP32, Arduino, etc.)

- Note: DHT11 is not super precise, but enough for monitoring room conditions. For higher accuracy, use DHT22/AM2302.

- DHT11 specifically to monitor the temperature and humidity inside the food and water compartment of your Health Assist Robot — so the food or water remains at a safe, ideal condition while being delivered to the patient.

- Trigger alerts if the compartment’s temperature exceeds safe limits (e.g., >30 °C).

## ⚙️ Ideal Placement in the Compartment

- To get accurate readings:

- 📍 Mount DHT11 midway inside the compartment (not touching the food/water directly).

- 🧊 Keep it shielded from steam or direct moisture — DHT11 is not waterproof. You can use:

  - A vented plastic housing or breathable mesh cover.

  - A silica gel moisture barrier nearby if the compartment is sealed.

- 🌡️ Avoid placing near heating elements or motor heat sources, or readings will be inaccurate.

(App interface for temp data etc.)

A Peltier module (also called a Thermoelectric Cooler, often marked TEC1-12706) is a small square plate that makes one side cold and the other side hot when you connect it to electricity.

- It’s based on something called the Peltier effect:

- When you pass electricity through special materials inside the module,
heat moves from one side to the other.

## Why Use It in Your Health Assist Robot?

- You can use the Peltier module inside your food and water compartment to:

- Keep water cool for the patient 🥤

- Keep food from heating up in a warm room 🍱

- Maintain medicine storage temperature 💊

- Basically, it’s like a mini refrigerator plate inside your robot!

## Important: It Gets VERY HOT!

- Because one side becomes cold, the other side becomes very hot (60–80 °C).

- So you must:

- Attach a metal heat sink (like computer CPU cooler)

- Use a cooling fan on the hot side to remove heat

- Otherwise, the Peltier burns out in minutes. 🔥

- We can’t let the ESP32 power the Peltier directly…
but we can let the ESP32 tell another big switch to turn the Peltier ON or OFF.

- That’s where the relay or MOSFET comes in.

- DHT11 measures temp → sends to ESP32

- ESP32 checks:

- If temp > 30°C → send HIGH to relay pin → Peltier ON

- If temp < 25°C → send LOW → Peltier OFF

- Compartment stays cool automatically ✅

DS18B20 is a digital temperature sensor made by Dallas/Maxim.
It accurately measures temperature in °C and sends the data digitally — meaning no analog voltage conversion is needed.

MG996R is a high-torque metal gear servo motor.
If a normal DC motor is like a spinning fan,
the MG996R is like a human arm joint — it moves to an exact angle and stays there.

## ⚡ Important: Power Warning

- Just like the Peltier, the servo draws more current than the ESP32 can provide.
So never power it directly from the ESP32 5V pin — it could reset or damage your board.

- ✅ Instead:

- Use an external 5–6V power supply (like a buck converter from your 12V battery).

- Connect grounds together (ESP32 GND ↔ servo GND).

- Don’t rotate the servo beyond 180°, it can break the gears.

MG90S is a mini metal gear servo motor.
Think of it as a small muscle — it can move parts precisely to specific angles (0° to 180°), just like a smaller version of MG996R.

FT232RL is basically the bridge between your computer (USB) and your ESP32, Arduino, or any microcontroller that talks using serial (TX/RX).

- The FT232RL is a USB to Serial (UART) converter chip made by FTDI.

- Think of it like a translator between:

- Your computer’s USB (which speaks in digital packets 💻)

- And your ESP32 or microcontroller’s serial pins (TX, RX) (which speak in UART 🗣️)

- You’ll use FT232RL if:

- You have an ESP32 bare chip or development board without a built-in USB interface (some ESP32-S modules don’t have one).

- You want to upload code, monitor serial output, or debug sensors through USB.

- You’re working with other UART devices (e.g., motor controllers, GPS, RFID modules) and need to connect them to your PC.

- You can connect FT232RL to any UART device (e.g., motor controller, GPS, etc.)
Then open the Serial Monitor on your PC to see what the device is sending.
Very useful for checking sensor data or debugging your robot’s systems.

Clamps hinges

- Food/Water compartment lid------Open/close lid with servo (MG90S)------- Mini metal or plastic hinge (2–3 cm)

- Medicine drawer flap----------Small rotating flap to drop medicine-------Micro hinge or plastic hinge

- Robot body panels------Access door for wiring, battery, etc.------ Metal hinge (stainless steel)

- Clamps are used to secure components — motors, sensors, batteries, or pipes — so they don’t move while the robot drives around.

- Mini hinge + MG90S servo → lid opens automatically.

- M3 screws to attach hinge and servo horn.

- Plastic clamp to hold DHT11 inside.

- Small flap fixed with hinge.

- Servo rotates flap → drops 1 pill.

- M2 screws and nuts for hinge and servo bracket.

### ⚡ Battery section:

- Velcro clamp → secure 12V battery.

- Metal hinge → door access to internal wiring.

### 1. Base & Wheels

- Start with the chassis/base plate (your acrylic sheet, 20×30 cm).

- Attach the 4 hub motors to the base corners.

  - Keep the wheels aligned straight.

  - Make sure the wheel overhang doesn’t interfere with compartments.

- This gives you a stable mobile platform before adding anything else.

### 2. Motor Driver & Power Circuit Placement

- Decide where your 4 cm “circuit” compartment will go (you mentioned 4 cm).

  - Place it near the center of mass so the robot doesn’t tip.

  - This compartment will house:

    - Motor drivers (H-Bridge or ESCs)

    - Microcontroller

    - Power supply

- Keep wires from motors to the driver as short as possible to reduce voltage drop

### 3. Compartment Placement

- Stack compartments from front to back according to your sizes:

  - Medicine dispenser (10 cm) – front or center.

  - Water & Food (12 cm) – next, slightly larger, could be center or back.

  - First Aid (6 cm) – either front/top depending on access.

- Make sure access doors or dispensers can open without hitting wheels or other compartments.

- Use screws or glue to secure compartments.

### 4. Microcontroller & Sensors

- Microcontroller goes in the circuit compartment.

- Any distance/IR sensors can be mounted at the front and corners for navigation.

- Servo motors for dispensing can be mounted inside the compartments.

### 5. Battery Placement

- Place the battery low and central to keep center of gravity stable.

- Secure it with a strap or tape inside the circuit compartment or below the base plate.

### 6. Wiring Planning

- Route motor wires first, then power wires, then control wires.

- Keep high-current motor wires separate from sensor/wire signals to avoid noise.

- Label wires if possible to make troubleshooting easier.